# SIMATS ENGINEERING

### ASSESSMENT TOOL 3

#### COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30% Software:Cisco Packet Tracer,Wireshark

#### QUESTIONS: 5	Total Marks:50

Annexure A

## Debugging & Optimisation Task

##### Code of Conduct:

I S.JAGADESH(192421406)  certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

##### Signature of the student:

Annexure A

### Debugging & Optimisation Task

- A network administrator has configured a workstation with the following settings:

  - IP Address: 192.168.10.65

  - Subnet Mask: 255.255.255.192 (/26)

  - Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Task

Debug the network configuration by:

  - Identifying the subnet to which the configured IP address belongs.

  - Determining the network address, broadcast address, and valid host range.

  - Verifying whether the configured IP address and gateway are valid.

  - Identifying the configuration errors.

  - Optimizing the configuration by assigning an appropriate IP address and default gateway.

  - Calculating the total number of usable host addresses after correction.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

- An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

  - Identifying the network address, broadcast address, and host range for each subnet.

  - Calculating the number of usable and unused IP addresses.

  - Identifying inefficient subnet allocations.

  - Debugging the addressing plan by locating subnet allocation mistakes.

  - Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails. Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

- A router contains the following routing table.

#### Destination Network	Next Hop

192.168.1.0/24	10.0.0.2

192.168.2.0/24	10.0.0.3

Default Route	10.0.0.1

Users report that packets destined for 192.168.2.100 are not reaching the intended network. Task

Debug the routing configuration by:

- Identifying the matching routing entry using the longest-prefix match.

- Determining the next-hop router selected for packet forwarding.

- Verifying whether the destination IP belongs to the intended subnet.

- Identifying any routing configuration errors.

- Optimizing the routing table to improve packet forwarding efficiency.

- Identifying the route that will be used if no matching destination exists.

4TH QUESTION DEBUGGING AND OPTIMIZATION

#### RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient ) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understandin g | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justificatio n | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentatio n | Code is mostly clear with minor documentat ion gaps | Code lacks structure and proper documentatio n | Poorly written code with no documentatio n |